MARANDA 2025 KCSE PREDICTION

EXPECTED EXAM 1

ELECTRICITY

448/1

PAPER 1 (Theory)

TIME: 2½ HOURS

NAME……………………………………………………………………………………..

SCHOOL………………………………………………   	SIGN………………………..

INDEX NO…………………………………………….	ADM NO……………………

Kenya Certificate of Secondary Education.

INSTRUCTIONS TO CANDIDATES

- Write your Name, Index number and Class in spaces provided

- Candidates should have the following for this examination

Drawing instruments

Scientific calculator

- This paper consists of two sections: A and B.

- Attempt ALL questions in section A and any FOUR in section B.

- All dimensions are in millimeters unless stated other wise

For Examiner’s use only

| Section | Questions | Maximum Score | Candidate’s Score |
| --- | --- | --- | --- |
| A | 1-10 | 48 |  |
| B | 11 | 13 |  |
|  | 12 | 13 |  |
|  | 13 | 13 |  |
|  | 14 | 13 |  |
|  | 15 | 13 |  |

SECTION A (48 marks)

(Attempt all questions in this section)

- Explain the significance of using only one hand when working on high voltage system														(2 marks)

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

- a)	Define each of the following terms:						(3 marks)

- semiconductor

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

- intrinsic semiconductor

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

- extrinsic semiconductor

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

b)	Draw and label voltage versus current characteristics curve for a P-N junction in 	its forward and reverse modes										(3 marks)

- a)	A 5kΩ linear potentiometer has 6V source connected across its end terminals

- Determine the range of output voltage of the potentiometer				(1 mark)

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

- Calculate the two possible output voltages when the potentiometer is in its three-quarter position												(2 marks)

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

b)	An emf of 120V causes a current of 3A to flow in a circuit. Calculate the energy 	supplied by the source in 12 seconds									(2 marks)

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

- a)	Outline two differences between a permanent magnet and an electromagnet	 (2 marks)

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

b)	State and explain two properties of a good conductor					(2 marks)

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

- a)	List four electrical measurements that can be taken using a cathode ray oscilloscope													(2 marks)

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

b)	A moving coil meter gives a full scale deflection for a current of 100mA and has a resistance of 5Ω. Determine the value of the resistor used so that the meter can measure a current of 1.0A at full scale deflection.									(3 marks)

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

- a)	Draw the current discharge curve of a capacitor					(2 marks)

b)	Outline how insulation resistance test can be carried out between two conductors	 (5 marks)

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

- a)	Name the logic gate shown in Figure 1				(1 mark)

Figure 1

Name…………………………………………..

b) Complete the truth table of the OR gate 			(2 marks)

| INPUT 1 | INPUT 2 | OUTPUT |
| --- | --- | --- |
| 0 | 0 |  |
| 0 | 1 |  |
| 1 | 0 |  |
| 1 | 1 |  |

c)	Perform each of the following number conversions					(4 marks)

- 1010102 to decimal

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

- 2310 to binary

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

- a)	Name four sources of electricity in Kenya					(2 marks)

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

b)	State one possible cause and remedy for each of the following electric motor faults														(4 marks)

- Motor connected to the supply but will not start

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

- Motor overheats on load

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

- Draw wiring diagram of a 12V electric bell controlled by two switches from a 240V mains supply											(4 marks)

- a)	Define entrepreneur									(1 mark)

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

b)	List two business opportunities in the field of electricity				(1 mark)

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

…………………………………………………………………………………………………………

SECTION B (52 marks)

Answer any four questions from this section in the spaces provided

- a)	State three variables which determine resonance condition in an RLC circuit (3 marks)

b)	A series circuit consisting of a 0.1µF capacitor and a 25µH, 20Ω coil is connected across a 10V a.c supply. Calculate the following:							(10 marks)

- Resonant frequency

- Supply current at resonance

- Voltage across the capacitor at resonance

- With the aid of a labelled diagram, show the construction and describe the operation of a moving coil permanent magnet type instrument					(13 marks)

- a)	Explain the term ideal transformer							(3 marks)

b)	A transformer which has a primary winding resistance of 1Ω and a secondary winding resistance of 0.1Ω is connected to a 10Ω load. if the primary current is 0.5A and secondary is 5A, calculate:											(6 marks)

- Load power

- Copper losses

c)	If the transformer in (b) has a core loss of 5W, calculate the efficiency of the transformer													(4 marks)

- The transistor in the circuit shown in Figure 2 below is biased to supply the relay with 100mA. the dc current gain, β, of the transistor is 50 and the forward base-emitter voltage, VBE is 0.7V

- If the voltage drop across the relay is 12V, calculate the:				(11 marks)

- Base current IB

- Emitter current IE

- Voltage between collector and emitter VCE

- The base resistance RB

- Explain the function of diode D1 in the circuit						(2 marks)

- Figure 3 shows three views of an object drawn in first angle orthographic projection. make isometric drawing of the object taking A as the closest point	(13 marks)

Figure 3